Cylinder Counting from Moving Truck

## 1. Background & Business Context

In industrial logistics (LPG, oxygen, industrial gas supply chains), trucks carry dozens to hundreds of cylinders daily.

2. Problem Statement

The objective of this assignment is to design a computer vision–based system that can automatically process a top-view video of a moving truck and accurately detect and count the gas cylinders loaded on it. The solution must ensure that each cylinder is counted exactly once while remaining robust to real-world challenges such as continuous vehicle motion and partial occlusion of cylinders as the truck moves through the camera’s field of view.

3. Objective

- To automatically detect gas cylinders from a top-view video of a moving truck

- To ensure accurate counting by preventing duplicate or double counting of cylinders

- To generate a reliable and consistent final cylinder count for the entire truck

- To provide visual validation of detections and counts through annotated video frames

## 4. Approach & Methodology

- YOLOv8 detects cylinders in each video frame

- A lightweight tracking mechanism assigns IDs

- A virtual counting line ensures cylinders are counted once

- Annotated frames show detections and counts

## 5. Technical Stack

Python

YOLOv8 (Ultralytics)

OpenCV

NumPy

filterpy (Kalman Filter dependency)

PyTorch

## 6. System Architecture

Input Video

↓

Frame Capture (OpenCV)

↓

Cylinder Detection (YOLOv8)

↓

Lightweight ID Assignment (Tracker)

↓

Counting Logic (Line Crossing)

↓

Annotated Output + Final Count

## 7. Project Workflow (Step-by-Step)

### 7.1 Video Input Handling

cap = cv2.VideoCapture(video_path)

- Reads frames sequentially

- Maintains frame order (important for counting)

7.2 Detection Using YOLOv8

model = YOLO("yolov8n.pt")

results = model(frame, conf=0.4)[0]

- YOLOv8n is used for:

    - Fast inference

    - Good generalization

- Confidence threshold = 0.4

- Outputs bounding boxes per frame

Each detection:

[x1, y1, x2, y2, confidence]

7.3 Detection Collection

detections.append([x1, y1, x2, y2, conf])

- These detections are passed to the tracker

- Clean separation between detection and tracking

8. Tracking & Counting Logic (Core Concept)

### 8.1 Tracker Design (sort.py)

class Sort:

def __init__(self):

self.track_id = 0

This tracker:

- Assigns a unique ID to each detection

- Does not perform motion prediction or matching

- Is a simplified tracking abstraction

self.track_id += 1

results.append([x1, y1, x2, y2, self.track_id])

8.2 Why Tracking Is Needed

Without tracking:

- The same cylinder appears in multiple frames

- It would be counted repeatedly

Tracking allows:

- Object identity persistence

- Counting logic based on unique IDs

9. Counting Logic (Very Important)

### Virtual Line Strategy

line_y = 300

A horizontal line is defined in the frame.

### Counting Condition

if cy > line_y and track_id not in counted_ids:

Explanation:

- cy = center Y of bounding box

- Cylinder is counted only once

- ID is stored in counted_ids

counted_ids.add(track_id)

total_count += 1

10. Visualization & Output

### Bounding Boxes & IDs

cv2.rectangle(...)

cv2.putText(frame, f"ID:{track_id}")

### Total Count Overlay

cv2.putText(frame, f"Total Count: {total_count}")

### Final Output

print("Final Cylinder Count:", total_count)

Outputs:

- Live annotated video

- Final count in terminal

11. Results & Observations

- Cylinders are detected reliably across most frames in the video

- Each cylinder is counted exactly once due to the ID-based counting logic

- Annotated video frames provide clear visual validation of detections and counts

- The system performs well under real-world conditions, including:

    - Moderate levels of partial occlusion

    - Continuous motion of the truck

    - A stable, fixed top-view camera setup